AD47

The station is Army Corps of Engineers brass cap on property owned by the City of Casa Grande at the Casa Grande landfill.

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), SE along 2nd Street 0.3 miles to Florence Street, Turn right (south) along Florence Street / ChuiChu Highway 2.9 miles to Canal crossing and N.E. corner of City landfill property. Station is Army Map Service brass disk in concrete post and target located inside landfill property 160’ south of canal, 125’ west of centerline of highway. Cap is marked  “AD  47”  “1968”

[image: image1.jpg]




AK45

Station is U.S. Army Corps of Engineers brass cap as described.

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), East on Florence Blvd. 6.0 miles to Overfield Road. Right  (south) on Overfield one mile to Early Rd. and the station. Station is Army Map Service brass cap in concrete post and target in the southeast corner of the intersection. 79’  south and 82’ east of the centerline intersection.. Cap is marked “AK 45”  “1967”

[image: image2.jpg]




CG 131

Station is City of Casa Grande brass cap in ADOT centerline monument well in City of Casa Grande right-of-way.

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), East on Florence Blvd. 1.0 mile to Trekell Road. Turn left on Trekell (north) 1.0 miles to Cottonwood Lane. Turn right on Cottonwood Lane (east) 0.5  miles to monument on left being the south ¼ corner of Section 16, T. 6 S., R. 6 E., G.& S.R.M. Monument is City of Casa Grande brass cap in standard ADOT monument well 6’ north of curb, and 10.5’ east of box culvert under Cottonwood Lane.

[image: image3.jpg]




CG-177

Station is City of Casa Grande brass cap located on El-Paso Gas right-of-way

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), North on Pinal Ave.3.53 miles to El Paso Natural Gas line crossing..  Monument is City of Casa Grande brass cap in concrete post flush with natural ground marked “CG-177” in concrete post  flush with natural ground 195 feet east of the centerline of Pinal Ave., 104.5’ east of right-of-way fence and 50’ north of El-Paso gas line markers. 2’ N.W, of a witness sign.

[image: image4.jpg]




ARENA

Station is City of Casa Grande brass cap located on undeveloped land in the NW/4 of Section 36, T 5 S., R 6 E., G&SRM.

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), east along Florence Blvd. 4.0 miles to Interstate 10. North on I-10,  4.0 miles to McCartney Rd. exit. East on McCartney Road 0.6 miles to Cox Rd. Turn left on Cox road (north) 1.0 miles to Woodruff Rd. Turn left on Woodruff (west) 1.0 miles to track road. Station if City of Casa Grande brass cap in concrete post flush with natural ground marked “STATIC STA. ARENA” . Station is 51’ south of centerline of Woodruff Road, and 68’ east of tract road.

[image: image5.jpg]




CG109-A

Station is 24.5’ of stainless steel rod driven to refusal in NGS standard monument located on City of Casa Grande property at “Rotary Park”.

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), east on Florence Blvd 1.0 miles to Trekell Road. Turn left on Trekell (north) 2.6 miles to Yucca Street. Turn south on east frontage road along Trekell road 220’ to Rotary Park. Station is stainless steel rod in monument well on south berm of park entrance. 7.5’ South of “Rotary Park” sign.

Station is 24.5’ of stainless rod driven to refusal in standard NGS monument marked “CG 109 A”

[image: image6.jpg]




CG AIR-1
Station is 13.5’ of stainless steel rod driven to refusal in standard NGS monument on property owned by City of Casa Grande (Public works / Airport)

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), North 4.6 miles on Pinal Ave to Centennial Rd. Turn left (west) 0.22 miles to Lear Ave. Turn Right (east) on Lear to end and City of Casa Grande complex at 3181 N. Lear.

 Monument is 13.5’ of stainless steel rod driven to refusal in standard NGS monument well marked “CG AIR-1”, Station is  37’ north of the northeast corner of parking lot (370’ north of center cul-de-sac Lear) and 25.3’ west of a chain link fence and 78’ south of the south edge of Tati Way.

[image: image7.jpg]




CG NW-1

Station is smooth brass cap located on Gila River Indian Reservation

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), North 7.8 miles on Pinal Ave. to Fawnee Lane . Turn left on Fawnee Lane (west) 0.2 miles to bend in road to NW. Follow Fawnee Lane NW 0.3 miles to Millrise Rd. and Gila River Indian Reservation boundary. West along Millrise 1.1 miles to end at Faldale Rd..Pack 375’ West along reservation boundary fence and 74’ north to station. Station is smooth brass cap flush in bedrock outcrop marked “CG NW-1”
[image: image8.jpg]




CG NMP

Station is Smooth brass cap located on property owned by City of Casa Grande(North Mountain Park).

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), East of Florence Blvd. 4.1 miles to Interstate-10.  North on I-10, 4.0 miles to McCartney Rd. exit. East on McCartney Road 0.6 miles to Cox Rd. Turn left on Cox road (north) 3.0 miles to Waverly Drive and end of pavement. Continue N and NE along dirt road 0.6 miles to Camden Dr and entrance to North Mountian Park. Left on Camden Dr (west) 0.1 miles to first right. NW along park road 0.3 miles to fork. Take left on park loop 0.3 miles to “Family Ramada # 8. Station is brass cap in high rock outcrop 60’ northeast of Ramada #8. Pack to top of highest rock. Station is 4.5’ below highest rock, in small valley 9.5’ west of highest rock. Brass cap flush marked “ CG-NMP”

[image: image9.jpg]




E 278

Station is USC&GS brass disk located on undeveloped land near the east ¼ corner of  Section 18, T.6 S., R. 6 E. G&SRM.

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), North on Pinal Ave. 1.5 miles to Viola Dr. Exit left to vacant area south of Airport Tavern. Station is USC&G brass disk in concrete pump base as described by NGS, 42.26’ SW of the SE corner of building 30.7’ west of Pinal Ave curb, and 1.7’ below the roadway elevation.

[image: image10.jpg]



HAY RM2

Station is USC&GS brass cap located in El-Paso gas line right-of-way

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), east Florence Blvd. 1.0 miles to Trekell Rd. Turn left on Trekell Rd. (north) 2.0 miles to Kortsen Rd., Turn right on Kortsen (east) 3.1 miles to end at I-10 right-of-way. Station is USGS standard brass disk as described at SW corner of El-Paso gas enclosure 100’ north of Kortsen, and 100’ west of I-10 right-of-way fence.

[image: image11.jpg]




Sierra
Station is 15.0’ of stainless steel rod in NGS monument located on City of Casa Grande property in the NE/4 of Section 27, T. 6 S., R. 6 E.,G&SRM

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), East on Florence Blvd. 2.45 miles to Arizola  Rd.  Turn right on Arizola (south) 0.32 miles to Sierra Pky. Turn left on Sierra Pky (east) 0.3 miles to end paving. Continue SE 0.1 mile around church complex to Park/Basin. Station is stainless steel rod driven to refusal in NGS standard monument on east edge of basin, 180’ north of the SNE corner of the top of basin, and 2’ west of the parking rail. 

[image: image12.jpg]




Signal Peak Azimuth Cap
Station is NGS brass cap as described in bedrock outcrop located on undeveloped property in the NE/4 of Section 31, T. 5 S., R. 7 E., G&SRM

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), ), East of Florence Blvd. 4.1 miles to Interstate-10.  North on I-10, 4.0 miles to McCartney Rd. exit. East on McCartney Road 0.6 miles to Cox Rd. Turn left on Cox road (north) 1.0 mile to Woodruff Rd. Turn right on Woodruff (east) 0.8 mile to curve to right. Pack 600’ East to top of ridge and station 3.5’ below highest point on ridge. 

[image: image13.jpg]




HOHOKAM
Station is a Smooth brass cap in siphon headwall on USBR (CAP) property near the center of Section 21, T. 6 S., R. 6 E., G&SRM

To reach the station:

From the intersection of State Hwy 287 (Florence Blvd.) and Hwy 387 (Pinal Ave), East on Florence Blvd. 8.0 miles to Signal Peak road. Turn left (north) 0.5 miles to CAP canal crossing. Turn left (west) 0.55 miles along the north side of CAP canal to west headwall of an E-W siphon. Station is Brass cap in SE angle point of headwall marked “HOHOKAM”  “2006” “LS 20358”

[image: image14.jpg]




“4”

Record NGS description is adequate.

[image: image15.jpg]



E 422

Record NGS description adequate

[image: image16.jpg]



